City of Federal Way HVAC Specifications: CITY HALL

Standard service: Preventive maintenance program and repairs.

- Bi-Annual inspections, maintenance and testing to be performed spring and fall of each year for all listed equipment and VAV boxes. (see attached equipment list for each facility)

- Filters replaced 1 time per year on all related VAV equipment and belts replaced as needed.

- Filters replaced 2 times a year on all air handlers and belts replaced as needed.

- Inspection check sheets to be used as minimum standards for each type equipment.           (see attached typical check sheets)

- All motors 5 hp or larger, to be meg ohmed annually.

- All compressors to have oil samples taken annually and analyzed by qualified lab.  Test results emailed to City of Federal Way, contract manager.

- Contractor must have ability to self perform; programming of Staefa Smart II Controllers using a Staefa Smart II Tool for City Hall.

- Contractor must have ability to self-perform, troubleshoot and repair existing equipment.

Optional service: Comprehensive program to cover standard preventative maintenance items above, plus parts and labor for all items in the list of inclusions below:

Inclusions:

- Compressor

- Refrigerant

- Belts for the blower

- Electrical switches and relays within the units

- Motors and pump bearings

- Blower shaft

- Blower bearings

- Blower

- Fans

- HVAC automation & control systems

- Pumps

- Repair of heat exchanger

- Relays

- Filters (all units including but not limited to; RTU, VAV, MAU, AHU, CU, etc.)

- Cleaning of all covered equipment

- Motors starters

Exclusions for Comprehensive program:

- Condensate pans

- Air and water balance

- Piping, hoses, valves and hangers

- Ductwork and diffusers

- Motor starter panels

- Condenser/Evaporator coils

- Replacement of heat exchanger

- Metal casing around the unit

- Lamp housing, casings and enclosures

- Power and control wiring

- Variable Frequency Drive

- Pre-existing conditions

- Overtime Labor